PX-451A Root Cause Analysis Report

Failure Analysis Report

Equipment

PX-451A

Incident

Unexpected Shutdown

------------------------------------------------

Primary Cause

Bearing overheating

Root Cause

Insufficient lubrication

Secondary Cause

Mechanical seal leakage

Contributing Factors

High vibration

Delayed maintenance

------------------------------------------------

Corrective Actions

Replace SKF6318 bearings.

Replace mechanical seal.

Perform dynamic shaft alignment.

Flush lubrication system.

Restart only after vibration is below

4.5 mm/s.

------------------------------------------------

Preventive Actions

Lubricate every 500 operating hours.

Monthly vibration monitoring.

Quarterly oil analysis.

Annual bearing replacement.

------------------------------------------------

Lessons Learned

Lubrication schedule must never be skipped.

Seal leakage should be addressed immediately.