# Maintenance Manual – Centrifugal Pump P-101

## Document Information

- Document ID: MM-P101-001

- Equipment Name: Centrifugal Pump P-101

- Department: Maintenance

- Revision: 1.0

- Prepared By: Maintenance Engineering Department

- Approved By: Plant Maintenance Manager

- Effective Date: 15 July 2026

# 1. Purpose

This manual provides preventive and corrective maintenance procedures for Centrifugal Pump P-101 to ensure reliable operation, maximize equipment life, and minimize unexpected downtime.

# 2. Equipment Details

| Parameter | Value |
| --- | --- |
| Equipment Tag | P-101 |
| Equipment Type | Horizontal Centrifugal Pump |
| Service | Process Water Transfer |
| Driver | 15 kW Electric Motor |
| Rated Flow | 120 m³/hr |
| Rated Head | 35 m |
| Operating Speed | 2900 RPM |
| Seal Type | Mechanical Seal |
| Bearing Type | Deep Groove Ball Bearing |

# 3. Safety Precautions

Before starting maintenance:

- Obtain work permit approval.

- Wear mandatory PPE:

  - Safety Helmet

  - Safety Shoes

  - Safety Glasses

  - Protective Gloves

- Isolate electrical supply.

- Lockout and Tagout (LOTO) the motor.

- Close suction and discharge valves.

- Drain the pump casing completely.

- Ensure zero pressure before opening any component.

# 4. Required Tools

- Adjustable wrench set

- Socket wrench set

- Allen key set

- Torque wrench

- Bearing puller

- Soft mallet

- Dial indicator

- Grease gun

- Alignment kit

- Cleaning brushes

- Lint-free cloth

# 5. Spare Parts

| Part | Recommended Stock |
| --- | --- |
| Mechanical Seal Kit | 2 |
| Bearing Set | 2 |
| Shaft Sleeve | 1 |
| Impeller | 1 |
| O-Ring Kit | 5 |
| Gasket Set | 5 |

# 6. Daily Inspection

Check:

- Pump vibration

- Motor temperature

- Bearing temperature

- Lubrication leakage

- Mechanical seal leakage

- Abnormal noise

- Suction pressure

- Discharge pressure

- Flow rate

Record all observations in the maintenance log.

# 7. Weekly Maintenance

- Clean pump exterior.

- Inspect foundation bolts.

- Verify coupling guard condition.

- Check lubrication level.

- Inspect seal leakage.

- Tighten loose fasteners.

- Check motor current.

# 8. Monthly Maintenance

- Inspect coupling alignment.

- Check bearing condition.

- Inspect impeller wear.

- Verify vibration readings.

- Clean suction strainer.

- Inspect foundation for cracks.

- Verify pressure gauge calibration.

# 9. Quarterly Maintenance

- Replace bearing grease.

- Inspect shaft runout.

- Inspect mechanical seal faces.

- Measure vibration trend.

- Inspect casing for corrosion.

- Verify motor insulation resistance.

# 10. Annual Overhaul

Perform the following:

- Isolate the equipment.

- Remove coupling.

- Remove motor.

- Disassemble pump casing.

- Remove impeller.

- Inspect shaft.

- Replace bearings if worn.

- Replace mechanical seal.

- Replace all O-rings and gaskets.

- Inspect wear ring clearance.

- Reassemble pump.

- Perform shaft alignment.

- Conduct hydrostatic leak test.

- Run no-load test.

- Restore equipment to service.

# 11. Lubrication Schedule

| Component | Lubricant | Frequency |
| --- | --- | --- |
| Bearings | Lithium EP2 Grease | Every 3 Months |
| Coupling | Multipurpose Grease | Every 6 Months |

Do not over-grease bearings.

# 12. Alignment Procedure

Acceptable limits:

- Angular Misalignment ≤ 0.05 mm

- Parallel Misalignment ≤ 0.05 mm

Recheck alignment after tightening foundation bolts.

# 13. Vibration Limits

| Condition | RMS Velocity |
| --- | --- |
| Good | < 2.8 mm/s |
| Acceptable | 2.8–4.5 mm/s |
| Warning | 4.5–7.1 mm/s |
| Critical | > 7.1 mm/s |

# 14. Troubleshooting Guide

| Symptom | Possible Cause | Corrective Action |
| --- | --- | --- |
| No flow | Closed suction valve | Open suction valve |
| Low discharge pressure | Worn impeller | Replace impeller |
| Excessive vibration | Misalignment | Realign pump and motor |
| Bearing overheating | Insufficient lubrication | Re-grease bearings |
| Seal leakage | Damaged mechanical seal | Replace seal |
| Excessive noise | Cavitation | Check suction conditions |
| High motor current | Pump overload | Inspect discharge line |

# 15. Maintenance Checklist

- Work permit obtained

- Electrical isolation completed

- LOTO applied

- Pump drained

- Bearings inspected

- Seal inspected

- Impeller inspected

- Coupling aligned

- Fasteners tightened

- Lubrication completed

- Test run completed

- Documentation updated

# 16. Maintenance Record

| Date | Technician | Work Performed | Hours | Remarks |
| --- | --- | --- | --- | --- |

# 17. Revision History

| Revision | Date | Description |
| --- | --- | --- |
| 1.0 | 15-Jul-2026 | Initial release |